October 1, 2003

JAMAICA:  Montego Bay Airport (IFC #  11353)





DRAFT

ENVIRONMENTAL ACTION PLAN




March 11, 2002


SCA

Environmental & Social Action Plan

	E&S Element
	Description
	Schedule

	1.  Subsurface Contamination 
	· As part of future construction activities at Siem Reap, develop and implement procedures to identify and eliminate risk posed by the presence of unexploded ordinance in areas where construction will take place;

· Remove and remediate visibly contaminated soils from around the old diesel tank farm at Siem Reap;

· Investigate whether petroleum hydrocarbon contamination has spread beyond the immediate area, and present a plan to remediate any additional soil or groundwater contamination discovered.
	· Prior to Technical Completion

· Prior to first disbursement

· Prior to first disbursement



	2.  Fuel/Hazardous Material Storage and Handling
	· Install impermeable concrete containment beneath the diesel tanks and associated unloading area serving the power generators at Siem Reap;

· Install impermeable concrete containment beneath the diesel storage tanks at Pochentong Airport.
	· Prior to first disbursement

· Prior to first disbursement



	3.  Potable Water Supply
	Before making treated groundwater available as a source of drinking water at Siem Reap, ensure its quality meets WHO drinking water standards.
	As applicable

	4.  Liquid Effluent Discharges
	Install a new wastewater treatment plant at Siem Reap.  Plant effluent shall not exceed the following World Bank maximum limits:

· BOD5 – 50 mg/l;

· COD – 250 mg/l

· Oil and grease – 10 mg/l;

· Total suspended solids – 50 mg/l;

· Ammonia nitrogen – 10 mg/l

· Coliform bacteria -- <400 MPN/100ml


	Prior to Technical Completion

	5.  Pochentong Airport Runway Resettlement
	In coordination with the Government of Cambodia and consistent with the terms of the Concession Contract, ensure a specific Resettlement Action Plan (RAP) has been prepared and implemented consistent with IFC/World Bank policy on involuntary resettlement (OP 4.30) to IFC's satisfaction.
	Prior to Runway Expansion.


SBailey
M:\Environment Files\Mainstream Projects\Cambodia\Cambodia Airport-21363\EAP.doc
October 2, 2003 11:35 AM
2
3

